Microprocessor Lab

Laboratory Activity No. 2

Binary Representation of 8 LEDs in TinkerCad

And Arduino Programming

Score

Submitted by:

Isidro, Cheilo Marie P.

Sat 10:00-1:00 / CPE 0412.1-1

Date Submitted

11-10-2023

Submitted to:

Engr. Maria Rizette H. Sayo

I.	Objectives

The objective of this laboratory acitivity is to investigate and demonstrate the binary representation of 8 Light Emitting Diodes (LEDs) in TinkerCad through the utilization of Arduino programming. This experiment aims to provide a comprehensive understanding of binary numbering systems and their application in controlling multiple LEDs, fostering proficiency in Arduino programming, and gaining practical experience with TinkerCad for virtual circuit simulation.

II.	Method/s

- 	Write a code and perform an Arduino circuit diagram of a Binary representation (decimal 0-255 using 8 LEDs)

Materials Needed:

- Arduino Uno

- 8 LEDs

- 8 220 Ohms Resistors

- Breadboard

- Jumper / Connecting Wires

- Computer with TinkerCad and Arduino IDE Installed

TinkerCad Simulation Link: https://www.tinkercad.com/things/cDmxS4mSppM-tremendous-maimu-uusam/editel?sharecode=daYChWtQPK0_dReiMEn4fDOkatGQ9LFKMuEN3MdYHJY

Figure 1: Simulation in TinkerCad of Binary Representation of 8 LEDS

Written Code:

// C++ code

//

int num;

void setup()

{

for (int pin = 2; pin <= 9; pin++) {

pinMode(pin, OUTPUT);

}

Serial.begin(9600);

Serial.println("Enter The Number Which You Wanna Show In Binary:");

}

void loop()

{

if (Serial.available() > 0) {

num = Serial.parseInt();

Serial.println(num);

}

for (int pin = 2; pin <= 9; pin++) {

digitalWrite(pin, (num & (1 << (pin - 2))) ? HIGH : LOW);

}

}

III.	Results

TinkerCad Simulation

Actual Simulation (Arduino IDE and Arduino UNO)

IV. Conclusion

In conclusion, the code effectively demonstrates how to control 8 LEDs using Arduino and binary representation. By inputting a number through the serial monitor, we can see a visual binary representation, where each LED corresponds to a binary bit. This experiment not only provides a clear understanding of binary encoding but also showcases how Arduino programming can be used to control hardware components. It's a fundamental step in understanding the relationship between digital numbers and the physical world, opening the door to more complex applications in electronics and programming.

References

Login | TinkerCad. (n.d.). Tinkercad. https://www.tinkercad.com/things/f8h8YCQuJGg-copy-of-binary-representation-by-leds/editel?tenant=circuits